MODULE -2

PROGRAMMING MSP430

1. Development Environment for Embedded Systems

Embedded systems development requires specialized environments that include hardware, software, and debugging tools tailored to the embedded hardware. A typical embedded development environment includes:

- Toolchain: This includes the compiler (for C or Assembly), linker, and debugger. A popular toolchain is GCC (GNU Compiler Collection), used with a cross-compiler for the target embedded platform.

- IDE (Integrated Development Environment): IDEs like Keil µVision, IAR Embedded Workbench, or Eclipse with plugins like GNU ARM Eclipse are commonly used.

- Debugging and Simulation Tools: Debugging tools like JTAG or SWD and simulators for the target hardware (e.g., Proteus, QEMU) are essential for running code on actual hardware or virtual environments.

- Real-time Operating Systems (RTOS): Some embedded systems use an RTOS like FreeRTOS, EmbOS, or ChibiOS to handle multitasking and scheduling.

2. Aspects of C for Embedded Systems

The C language is widely used in embedded systems for its balance of control and abstraction. Some key aspects when using C for embedded systems:

- Low-Level Access: C provides direct access to memory, which is essential for embedded systems.

- Portability: C allows the code to be portable across different platforms, making it easier to scale embedded systems to different microcontrollers.

- Efficient Memory Management: Embedded systems often have limited resources, so developers need to use memory efficiently (e.g., avoid dynamic memory allocation).

- Optimization: Writing optimized code is crucial for performance in embedded systems.

Example in C for Embedded Systems:

#include <avr/io.h> // Header for AVR microcontroller

#include <util/delay.h> // Delay function

void initLED() {

DDRB |= (1 << PB0); // Set PB0 as output

}

void toggleLED() {

PORTB ^= (1 << PB0); // Toggle PB0 (LED)

}

int main() {

initLED(); // Initialize LED

while(1) {

toggleLED(); // Toggle the LED

_delay_ms(500); // Delay for 500 ms

}

return 0;

}

3. Assembly Language for Embedded Systems

- Assembly Language provides direct control over hardware and is often used in situations requiring optimization or direct interaction with hardware peripherals.

- It is hardware-specific and written for a particular processor architecture (e.g., ARM, AVR).

- Advantages: Speed, small memory footprint, fine control over processor resources.

- Disadvantages: Complexity, harder to maintain.

Example Assembly Code (AVR):

; Turn on an LED connected to pin PB0 on an AVR microcontroller

LDI     R16, 0x01      ; Load immediate value 0x01 into register R16

OUT     DDRB, R16      ; Set DDRB register to configure PB0 as output

LOOP:

SBI     PORTB, 0    ; Set bit 0 of PORTB (turn on LED)

NOP                 ; No operation (delay)

CBI     PORTB, 0    ; Clear bit 0 of PORTB (turn off LED)

NOP                 ; No operation (delay)

RJMP    LOOP        ; Jump back to LOOP

4. Register Organization in Embedded Systems

In microcontrollers, registers are used to hold data and control various hardware components. Registers can be classified as:

- General-purpose registers: Used for storing data temporarily during execution.

- Special function registers (SFRs): Used for configuring hardware peripherals (e.g., timers, interrupts).

Common registers include:

- Program Counter (PC): Holds the address of the next instruction.

- Status Register (SR): Holds flags (e.g., carry, zero, etc.).

- Stack Pointer (SP): Points to the top of the stack.

Example (AVR):

; Load value into a register

LDI     R16, 0xFF      ; Load 0xFF into register R16

5. Addressing Modes in Embedded Systems

Addressing modes define how operands (values) are accessed during instruction execution. Common addressing modes include:

- Immediate Addressing: Operand is provided as a constant in the instruction (e.g., LDI).

- Direct Addressing: Operand is in a specific memory location (e.g., MOV R0, 0x20).

- Indirect Addressing: Operand is accessed through a pointer stored in a register (e.g., LD R0, (R1)).

- Indexed Addressing: Combines a base address with an index (e.g., LD R0, 0x10(R1)).

6. Constant Generators and Emulated Instructions

- Constant Generators: These are built-in circuits in microcontrollers that generate specific constant values (e.g., power-of-2 values) without needing to use actual code to calculate them.

- Emulated Instructions: Some complex instructions may not exist in the instruction set, so they are emulated using multiple simpler instructions.

7. Instruction Set of Embedded Processors

- RISC (Reduced Instruction Set Computing): Uses a small, highly optimized instruction set (e.g., ARM Cortex-M).

- CISC (Complex Instruction Set Computing): Uses a larger set of instructions (e.g., x86 architecture).

Example Instruction Set (ARM):

- MOV: Move data between registers.

- ADD: Add two values.

- CMP: Compare two values.

8. Example Programs for Embedded Systems

- Light an LED: This simple program turns on an LED connected to a microcontroller.

#include <avr/io.h>

int main() {

DDRB |= (1 << PB0); // Set PB0 as output

while(1) {

PORTB |= (1 << PB0); // Turn on LED

}

return 0;

}

- Read Input from a Switch: A program to read a switch input and perform an action.

#include <avr/io.h>

int main() {

DDRC &= ~(1 << PC0); // Set PC0 as input (switch)

DDRB |= (1 << PB0);  // Set PB0 as output (LED)

while(1) {

if (PINC & (1 << PC0)) { // If switch is pressed

PORTB |= (1 << PB0); // Turn on LED

} else {

PORTB &= ~(1 << PB0); // Turn off LED

}

}

return 0;

}

- Flashing Light with Delay: Example of using a simple delay function to flash an LED.

#include <avr/io.h>

#include <util/delay.h>

int main() {

DDRB |= (1 << PB0); // Set PB0 as output

while(1) {

PORTB |= (1 << PB0); // Turn on LED

_delay_ms(500);      // Delay for 500ms

PORTB &= ~(1 << PB0); // Turn off LED

_delay_ms(500);      // Delay for 500ms

}

return 0;

}

9. Interrupts and Low Power Modes

- Interrupts: Interrupts are used to handle asynchronous events (e.g., a button press or timer overflow). When an interrupt occurs, the current execution is paused, and the interrupt service routine (ISR) is executed.

- Low Power Modes: Embedded systems often need to conserve power. Microcontrollers typically have several low-power modes:

  - Sleep Mode: The CPU is halted, but peripherals can still run.

  - Idle Mode: The CPU runs at a reduced clock speed.

  - Power Down: The CPU and most peripherals are disabled.

Example of Interrupt Handling:

ISR(INT0_vect) {

// Interrupt service routine for INT0

PORTB ^= (1 << PB0); // Toggle LED

}

int main() {

DDRB |= (1 << PB0); // Set PB0 as output

EIMSK |= (1 << INT0); // Enable external interrupt INT0

sei(); // Enable global interrupts

while(1);

return 0;

}