Module 4:

COMMUNICATION PERIPHERALS & PROTOCOLS

1. Communication Interfaces in Embedded Systems

Embedded systems often communicate with external devices, such as sensors, displays, or other microcontrollers, using standard communication protocols. The MSP430 microcontroller supports a range of communication interfaces for connecting with external devices.

1.1 USART (Universal Synchronous Asynchronous Receiver/Transmitter)

The USART module is used for serial communication and can operate in both synchronous and asynchronous modes. It is primarily used to transmit and receive data bit by bit over a single wire or pair of wires.

- Asynchronous Mode: Data is transmitted without a clock, typically using start and stop bits to frame the data.

- Synchronous Mode: Data is transmitted with a clock signal, which ensures that both sender and receiver are synchronized.

Key Features:

- Full-duplex communication (simultaneous send and receive).

- Typically used in UART (Universal Asynchronous Receiver/Transmitter) and SPI (Serial Peripheral Interface).

Example (MSP430 USART - Asynchronous Mode):

#include <msp430.h>

void USART_init() {

P1SEL |= BIT1 + BIT2; // Configure P1.1 as RX and P1.2 as TX

UCA0CTL1 |= UCSWRST; // Put USART in reset

UCA0CTL1 = UCSSEL_2; // Use SMCLK

UCA0BR0 = 104; // Set baud rate to 9600

UCA0BR1 = 0;

UCA0MCTL = UCBRS_1; // Set modulation for 9600 baud rate

UCA0CTL1 &= ~UCSWRST; // Release USART from reset

}

void USART_transmit(char data) {

while (!(IFG2 & UCA0TXIFG)); // Wait for TX buffer to be ready

UCA0TXBUF = data; // Send data

}

int main() {

USART_init(); // Initialize USART

while (1) {

USART_transmit('A'); // Transmit character 'A'

__delay_cycles(1000000); // Delay

}

return 0;

}

1.2 USCI (Universal Serial Communication Interface)

The USCI module is more advanced and provides support for UART, SPI, and I2C communication. It can operate in both synchronous and asynchronous modes and is available in newer MSP430 devices.

- Supports multiple protocols: USART (UART), I2C, and SPI.

- Flexible configuration for different baud rates, data formats, and clock settings.

Example (MSP430 USCI - SPI Mode):

#include <msp430.h>

void USCI_SPI_init() {

P1SEL |= BIT5 + BIT6 + BIT7; // Assign pins for SCLK, MOSI, MISO

UCB0CTL1 = UCSWRST; // Put USCI in reset

UCB0CTL0 = UCCKPL | UCMSB | UCMSTR | UCSYNC; // SPI mode, MSB first, synchronous

UCB0CTL1 = UCSSEL_2; // Use SMCLK as clock source

UCB0BR0 = 2; // Set baud rate (prescaler)

UCB0BR1 = 0;

UCB0CTL1 &= ~UCSWRST; // Release USCI from reset

}

void SPI_transmit(char data) {

while (!(IFG2 & UCB0TXIFG)); // Wait for TX buffer to be ready

UCB0TXBUF = data; // Send data

}

int main() {

USCI_SPI_init(); // Initialize SPI

while (1) {

SPI_transmit('A'); // Transmit data

__delay_cycles(1000000); // Delay

}

return 0;

}

1.3 USI (Universal Serial Interface)

The USI module is a simpler interface compared to USCI and is available on older MSP430 models. It supports basic I2C and SPI communication. USI is less flexible than USCI, as it only supports a subset of the serial communication protocols.

- Primarily used for I2C and SPI.

- Typically suitable for simpler applications requiring serial communication.

2. Communication Protocols in Embedded Systems

2.1 SPI (Serial Peripheral Interface)

SPI is a high-speed synchronous serial communication protocol used for connecting a microcontroller to peripherals like sensors, memory devices, and displays. It uses four lines:

- MOSI (Master Out Slave In): Data from master to slave.

- MISO (Master In Slave Out): Data from slave to master.

- SCLK (Serial Clock): Clock signal generated by the master.

- SS (Slave Select): A signal indicating which slave device the master is communicating with.

Key Features:

- Full-duplex communication.

- Supports multiple slaves (using different chip select lines).

- High-speed data transfer.

Example: SPI Master Sending Data (MSP430 with USCI)

// Already shown in the previous code snippet for USCI_SPI_init().

2.2 I2C (Inter-Integrated Circuit)

I2C is a widely used communication protocol for short-distance communication between devices. Unlike SPI, I2C uses only two wires:

- SDA (Serial Data): The data line, shared between devices.

- SCL (Serial Clock): The clock line, also shared.

Key Features:

- Multi-master support: Multiple devices can act as master.

- Slave addressing: Devices are addressed using a unique address.

- Two-wire interface: Reduces the number of wires needed for communication.

Example (MSP430 I2C - USCI Mode):

// Initialize I2C (USCI)

#include <msp430.h>

void I2C_init() {

P1SEL |= BIT6 + BIT7;  // Configure pins for SDA and SCL

UCB0CTL1 = UCSWRST;  // Put USCI in reset

UCB0CTL0 = UCMSTR | UCMODE_3 | UCSYNC; // Master mode, 7-bit addressing

UCB0CTL1 = UCSSEL_2; // Use SMCLK

UCB0BR0 = 10;  // Set baud rate

UCB0BR1 = 0;

UCB0CTL1 &= ~UCSWRST; // Release USCI from reset

}

void I2C_transmit(unsigned char data) {

while (!(IFG2 & UCB0TXIFG)); // Wait for TX buffer to be ready

UCB0TXBUF = data; // Send data

}

int main() {

I2C_init();  // Initialize I2C

I2C_transmit(0x55);  // Send data

return 0;

}

2.3 USB (Universal Serial Bus)

USB is a widely used protocol for communication between computers and peripheral devices. It is designed to support plug-and-play functionality and high-speed data transfer.

- Full-duplex communication: Allows data to be transmitted and received simultaneously.

- Master-slave architecture: The host (usually a computer) controls the communication, while peripherals (e.g., keyboards, mice) are slaves.

Key Features:

- Supports high-speed data transfer.

- Power delivery to peripherals.

- Plug-and-play functionality.

2.4 CAN (Controller Area Network)

CAN is a robust communication protocol designed for embedded systems in automotive and industrial applications. It is widely used for real-time control and communication between microcontrollers, sensors, actuators, and other devices.

- Multi-master: Allows multiple devices to initiate communication.

- Message priority: CAN provides message prioritization based on ID, making it ideal for real-time systems.

- Error detection: CAN includes strong error detection mechanisms, ensuring data integrity.

Key Features:

- Multi-master support.

- High error detection and correction capabilities.

- Real-time and fault-tolerant communication.

3. Summary

- USART is used for asynchronous and synchronous serial communication.

- USCI is a more advanced interface than USART, capable of handling UART, SPI, and I2C protocols.

- USI is a simpler version of USCI and supports basic serial communication.

- SPI is used for fast data transfer between devices, using four lines: MOSI, MISO, SCLK, and SS.

- I2C is a two-wire protocol, widely used in embedded systems for communication with multiple devices.

- USB is a universal protocol designed for data exchange between peripherals and computers, supporting high-speed communication.

- CAN is used in critical applications requiring high-speed, real-time communication, typically in automotive or industrial environments.